MONITOR

Rev 1.1

Release Date

September 28,1989

******** DISCLAIMER ********

The FBUG monitor is the sole property of Motorola Inc. This software may be

distributed in whole or in part as public domain software. This software is

provided as is and in no event shall Motorola, Inc. be liable for any direct,

special, incidental, or consequential damages arising out of or connected with

a users poseesion or use of this software package, even if Motorola has advance

notice of the possibility of such damages.

Comments, questions or suggestions may be directed, in writing only, to:

Microprocessor Products Applications

6501 William Cannon Drive West 0E33

Austin, Texas 78735-8598

*****************************

TABLE OF CONTENTS

1.      GENERAL INFORMATION

1.1     Description of FBUG.......................      3

2.      THE FBUG COMMAND SET

2.1     Introduction..............................      4

2.2     Assembler/Disassembler (as) ..............      6

2.3     Block of Memory Fill (bf).................      6

2.4     Block of Memory Move (bm).................      6

2.5     Break Point (br)..........................      6

2.6     Block Search (bs).........................      7

2.7     Confidence Test (ct)......................      7

2.8     Data Conversion (dc)......................      7

2.9     Go (go)...................................      7

2.10    Help (?/he/help)..........................      8

2.11    Load S-Records (lo).......................      8

2.12    Memory Display (md).......................      8

2.13    Memory Modify (mm)........................      9

2.14    Register Display (rd).....................      10

2.15    Register Modify (rm)......................      10

2.16    Symbol Define (sd)........................      10

2.17    Transparent Mode (tm).....................      10

2.18    Trace (tr)................................      11

3       USING THE ONE-LINE ASSEMBLER/DISASSEMBLER

3.1     Introduction  ............................      12

3.2     Entering and Modifying Source Program ....      12

3.3     Entering a Source Line....................      12

3.4     Entering Branch and Jump Addresses........      14

3.5     Entering Register Lists ..................      15

3.6     Entering Floating Point Immediate Data ...      15

3.7     Entering MMU Instructions ................      15

4       SOFTWARE MODIFICATIONS

4.1     Introduction ............................. 	16

4.2     Adding/Deleting Commands ................. 	16

4.3     Switching Drivers ........................ 	17

5       TARGET SYSTEM CONFIGURATION

5.1     Introduction ............................. 	17

5.2     Assembler/Disassembler ................... 	18

5.3     Memory Requirements ...................... 	18

6	LINKING INFORMATION

6.1	Introduction .............................	19

6.2	Description of FBUG (68xxx) Makefile .....	19

6.3	Monitor used to boot the system ..........	19

7	ERROR REPORTING

7.1	Error Listing .............................	19

CHAPTER 1

GENERAL INFORMATION

1.1     DESCRIPTION OF FBUG(68xxx)

The FBUG monitor is a stand alone software package designed to assist in

evaluating/debugging systems which use the 68xxx Microprocessor.  It has the

capability to load and execute user code and includes an assembler/disassembler

designed for quick program patchwork.  The monitor operates in a user-

interactive command driven mode signified by the MOTOROLA> prompt.  The

command line entered after this prompt determines which operation is

performed.

CHAPTER 2

THE FBUG COMMAND SET

2.1     INTRODUCTION

This chapter explains the FBUG monitor commands and their associated syntax.

Table 2.1 summarizes the available commands and shows the section where the

command is explained in greater detail.

TABLE 2.1  FBUG MONITOR COMMANDS

Command Mnemonic        Name                    Section

as      Assembler/Disassembler          2.2

bf      Block of Memory Fill            2.3

bm      Block of Memory Move            2.4

br      Breakpoint                      2.5

bs      Block of Memory Search          2.6

ct      Confidence Test                 2.7

dc      Data Conversion                 2.8

go      Go                              2.9

?/he/help      Help                            2.10

lo      Load S-Records                  2.11

md      Memory Display                  2.12

mm      Memory Modify                   2.13

rd      Register Display                2.14

rm      Register Modify                 2.15

sd      Symbol Define                   2.16

tm      Transparent Mode                2.17

tr      Trace                           2.18

The command line is composed of:

<COMMAND IDENTIFIER>: specifies which command (ex. br )

<SP>: at least one space

OPTION LIST: an option delimiter(-) with options if non-default options

are allowed and are being used. (ex. [<-r>])

<SP>: at least one space

ARGUMENTS: any required arguments specified by the command separated

by commas/spaces as shown in the command description.

(ex. <ADDR,ADDR>)

where "<>" enclose symbols that are required on the command line and "[<>]"

enclose symbols that are optional on the command line.  Note, in the above

examples the -r option was an example of an optional symbol and that the ADDR

fields are requirements on the command line.  The options available with a given

command are fully explained in the section that describes that command.  The

monitor is not case sensitive to input from the terminal. All input from the

terminal is converted to lower case before being used internally (except text

following a text delimiter; See TEXT below).  The arguments of a given command

are described using the following symbols:

<EXP>: An expression can be any numerical expression which may be

evaluated using only the arithmetic + and - operators.

Ex.     1000

Ex.     1+3

Ex.     13+/reset   NOTE: where /reset has been

predefined (see section 2.16 )

Note: Numbers may be preceded with a base designator if the default

(hexadecimal) is not desired. These designators are shown below in

Table 2.2:

TABLE 2.2  BASE DESIGNATORS

_______________________

Base         Designator

_______________________

Hexadecimal     $

Decimal         &

Octal           @

Binary          %

<ADDR>: Address field is any valid expression. Note: This address field should

not be confused with the source and destination addresses required

using the Assembler/Disassembler.

<COUNT>: Count field is any valid expression preceded by a COUNTDEL (count

delimiter ie. ":")

Ex.     :100

<RANGE>: A range of memory locations denoted by either ADDR,ADDR or ADDR:COUNT.

Ex.     0,100

Ex.     0:50

<TEXT>: An ASCII string of up to 255 characters preceded by a TEXTDEL (text

delimiter ie. ";")

Ex.     ;sample text

<SIZE>: Can be either:

byte    (8   bit)       ====> -b

word    (16  bit)       ====> -w

long    (32  bit)       ====> -l

Note: ====> stands for "is represented by" or "returns"

<DATA>: Data can be any valid expression.

<MASK>: A mask may be any expression.  After evaluating the expression 0's

represent don't cares.  A mask is sometimes used to qualify

<DATA>. See section 2.6 for an example of usage.

2.2     ASSEMBLER/DISASSEMBLER

as <ADDR>

The assembler/disassembler is invoked at the address given and disassembles

the object code at that location.  Use of the Assembler/Disassembler is fully

described in chapter 3.

2.3    BLOCK OF MEMORY FILL

bf [<SIZE>] <RANGE> <DATA>

The block fill command fills the specified range of memory with the data listed.

If the size option is not specified the default size used is word.  If a

multiple of the <SIZE> of <DATA> does not fit evenly in the <RANGE> the command

leaves the last partial word or long word unchanged.

Examples of use:

bf 100,110      &10

bf 100:8        &10

bf -w 100:8     a

bf -l 100,110   a000a

Note: All of these examples perform the same memory fill.

(ie.  $00000100: $000a $000a $000a $000a $000a $000a

$0000010C: $000a $000a $0000 $0000 $0000 $0000 )

2.4     BLOCK OF MEMORY MOVE

bm [<SIZE>] <RANGE> <ADDR>

The block move command allows the user to copy segments of memory to different

locations in memory. The execution of this command does not destroy the original

version unless the location moved to <ADDR> is within the range <RANGE> of the

code being copied.  The size option is only available when range is described as

<ADDR>:<COUNT> .  If range is being described with the <ADDR>,<ADDR>  mode the

size defaults to byte. The size field represents the size transfer that is used

to accomplish the memory move.

Examples of use:

bm 1000,2000    10000

bm 1000:800     10000

bm -l 1000:400  10000   Note: This variation executes the fastest

Note: All of these examples perform the same memory move.

2.5     BREAKPOINT

br

br <ADDR>

br <ADDR> <:COUNT>

br -r [<ADDR>]

br -r

The breakpoint command allows the user to list, insert or delete breakpoints in

the target code. This allows the user to stop executing a program and return to the monitor environment when the specified <ADDR> is prefetched.  The different

uses of this command are summarized below:

br                              list all known breakpoints

br <ADDR>                       insert a breakpoint at this address

br <ADDR> <:COUNT>              insert a breakpoint at this address

,however, return to the monitor

environment only after encountering the

breakpoint <COUNT> number of times.

br -r [<ADDR>]                  remove the breakpoint at this address

br -r                           remove all breakpoints

2.6     BLOCK SEARCH

bs [<SIZE>] <RANGE> <DATA>

bs [<SIZE>] <RANGE> <DATA> <MASK>

The block search command allows the user to find a specific pattern within

memory.  The search area may extend beyond the <RANGE> specified if a pattern is

started within <RANGE>.  There are two primary types of searches:

bs [<SIZE>] <RANGE> <DATA>              searches the range for an

exact match of <DATA>.

bs [<SIZE>] <RANGE> <DATA> <MASK>       searches the range for any

pattern that matches <DATA>

where there is a "1" in the

binary representation of

the mask.

Ex. With memory at location $100 as shown below, executing

"bs 100,118 $1234 $ffbf" ====>

Starting address: $00000100

Ending   address: $00000117

Found at: $00000110:$1234

Found at: $00000114:$1274

Memory for the example above:

$00000100: $0000 $0000 $0000 $0000 $0000 $0000

$0000010C: $0000 $0000 $1234 $0000 $1274 $0000

2.7     CONFIDENCE TEST

ct

The confidence test command allows the user to perform a confidence test of the

system being evaluated. This command is designed to write a -1 into the result

variable (int confres in vardef.h) of the confidence test and then pass the

address of this variable in register a0 to a called subroutine. This routine

writes a 0 or 1 into confres depending on whether the test passes or fails

respectively. If the routine does not place a valid result in this variable

then the monitor assumes that the confidence test was not performed and displays

"ERROR 14:Unable to perform Confidence Test".  The confidence test to be

performed is system dependent and has to developed for the system on

which the monitor is running.

2.8     DATA CONVERSION

dc <EXP>

The data conversion command allows the user to evaluate an input expression and

determine its hexadecimal and decimal equivalent.

Examples of use:

NOTE:   The following symbols have been defined earlier in order

fpr them to be used in the examples below:

EX 1.   uses	/start= 0

Ex 2.   uses	/start= - $18

Ex 3.   uses	/finish= 10000 and /start=$10000

(see section 2.17 )

Ex. 1   dc $17+/start      ====>   $17 = &23

Ex. 2   dc $17+/start      ====>   UNSIGNED  :  $FFFFFFFF = &4294967295

SIGNED    :        -$1 = -&1

Ex. 3   dc $/finish-/start ====>   $10000 = 65536

2.9     GO

go [<ADDR>]

The go command allows the user to execute target programs. If an address is not

specified on the command line then the current PC value is used.This value is

either 1.)the initialized PC value if no target code has been run,2.)the last

value of the PC used in executing target code or 3.)the value placed into the PC

register by a RM command (Register Modify see section 2.16 ). If an address is

included on the command line then the PC is modified to be the specified addr.

and execution begins at this address.  In both cases, the register state that

the microprocessor is intialized to, before executing the target code at this

location, can be viewed by executing a rd command (See section 2.15).

2.10    HELP

?    [<symbol>]

he   [<symbol>]

help [<symbol>]

The help command allows the user to view a list of allowable commands and the

syntax associated with them.  Symbols used to describe the command usage can be

looked up also.

Examples of use:

?,he or help ====>  return a complete listing of all commands with usage

? as         ====>  AS <addr>

help addr    ====>  <number>

he number    ====>  <hex> || <dec> || <oct> || <bin> || <symbol>

Note: <number> may also be an expression

2.11    LOAD S-RECORD

lo [<OFFSET>] ;<TEXT>

The load command allows the user to download S-Records from the host system.  If

an offset is present on the command line then the target address is the offset

added to the address determined by the S-Record. This command sends the <TEXT>

beyond the ";" to the HOST.  It then expects the HOST to begin sending S-Records

to the terminal.

Examples of use:

lo ;cat fbug.mx

Note:   The "cat" command is a UNIX command that concatenates and then prints

the specified files using standard output. This effectively sends

the contents of the file to the terminal. The monitor then loads the

contents of the S-Records in the file to the addresses determined by

the S-Records.

lo a0000 ;cat fbug.mx

Note: 	This command downloads the same S-Record file used

in the first example except that it is down loaded

into memory at the address determined by the

S-Record + $a0000 (ie. the offset is added in).

2.12    MEMORY DISPLAY

md [<SIZE>]  <addr>

md [<SIZE>]  <RANGE>

md -di <addr>

The memory display command allows the user to view memory.  The size used to dis

play the memory is determined by the size option.  If no option is used the

default is word.  If the range exceeds the screen capacity, output to the screen

is suspended until any key is pressed.

Examples of use:

md -l 100,110

md -l 100:4

md 100:8

md -di 100	Note: This command begins to disassemble the memory at

this location.

2.13    MEMORY MODIFY

mm [<SIZE>]  <ADDR>

mm <CONTROL>

The memroy modify command allows the user to view and modify memory.  The size

used to display the memory is determined by the size option.  The size default

is word.  Memory is displayed beginning at the address specified followed by a

'?' prompt.  The user may type in an <exp> to replace that memory value or hit

return to view the next memory value.  To exit the command, type ". <cr>"

(period <carriage return>).  Other available <CONTROL> characters are summarized

below in Table 2.3:

TABLE 2.3  CONTROL CHARACTERS

__________________________________

Control Character       Designator

__________________________________

- <EXP>                 backup  <EXP> memory locations

+ <EXP>                 advance   "     " 	"

Examples of use:

mm -l 100 ====> $00000100 $00000000 ?

mm 100    ====> $00000100 $0000 ?

(ie. uses the default "word" size)

2.14    REGISTER DISPLAY

rd

rd -f   Note: Only valid if Coprocessor support has been specified

see below.

The register display command allows the user to view the contents of the

registers of the mpu/fpu.  Versions released before August 30,1989 only

supported the 68020 programmers model and did not support the coprocessor

(68881/882) programmers model.  Versions released after August 30,1989 support

the programmers model that the user specifies before compiling the monitor.

This is done by using the DEVICE,EMULATOR and COPROCESSOR "define" statements in

the "targetsys.h" header file. These defines are discussed further in the

"Target System Configuration" section. The values shown for the registers are

either 1.) the initialized register state if no target code has been run, 2.)

the last values in the registers  while executing target code or 3.) the values

specifically placed into the registers by a rm command.

2.15    REGISTER MODIFY

rm [<REGISTER> [<New Value>]]

The RM command allows the user to the modify the contents of the registers of

the mpu.

Examples of use:

To change the PC value:

rm pc 3000      ====>   changes the PC value to 3000

or

rm              ====>   Which register?

pc              ====>   PC=00004000NEW VALUE?

3000            ====>   changes the PC value to 3000

or

rm pc           ====>   PC=00004000NEW VALUE?

3000            ====>   changes the PC value to 3000

2.16    SYMBOL DEFINE

sd [<SYMBOL>  <EXP>]

sd -r <SYMBOL>

The symbol define command allows the user to define symbols.  These symbols can

then be used within expressions. Using a symbol in an expresion results in the

symbol being substituted with the expresion that was used to define it.  Once

defined, the symbol is available until the monitor is reset.  If a symbol is

defined multiple times the monitor uses the first definition.

Examples of use:

sd              ====>   lists which symbols are already

known

sd /reset 10000 ====>   defines /reset to be $10000

whenever it is used in an expression.

sd /start -$18  ====>   defines /start to be -$18

whenever it is used in an expression.

sd -r /start    ====>   removes the first definition of /start

from the list

NOTE:   Symbols that have been defined using the sd command can

be used in any expressions. An example of this is to

use a symbol defined to enter source code while in

the assembler (ie.  bsr    /startsub     after defining

/startsub).

2.17    TRANSPARENT MODE

tm

The transparent mode command places the user into transparent mode by

establishing a software connection between the HOST and TERMINAL.  Transparent

mode preempts normal communication between the TERMINAL and the debugger. While

in this mode all keyboard input is relayed directly to the HOST. HOST responses,

in turn, are returned to the screen. Typing a CTRL A returns the user to the

monitor environment.

2.18    TRACE

tr [<ADDR>][<COUNT>]

The trace command allows the user to trace though target code and observe the

registers after executing the command line.  If count is specified then the

microprocessor executes <COUNT> number of instructions before returning to the

monitor environment.  Trace begins from the <ADDR> listed on the command line or

from the current PC if an <ADDR> is not included.  The trace instruction can be

continued by hitting a carriage return.  To exit, a "." must be entered.

Examples of use:

tr              ====>  traces 1 instruction from the current PC

tr :10          ====>  executes 10 instructions past the current

PC then returns to the monitor

environment

tr 1000         ====>  traces 1 instruction starting at $1000

CHAPTER 3

USING THE ONE-LINE ASSEMBLER/DISASSEMBLER

3.1     INTRODUCTION

Included in the FBUG monitor is an assembler/disassembler command which can

be executed as detailed in the previous chapter.  This assembler/disassembler

allows the user to modify target code.  Each source line that is typed in by the

user is entered into memory at the displayed address.  This line is then

disassembled so that the user can verify the actual code entered into memory. If

no change is desired a <CR> moves the user to the next opcode in memory.

NOTE:   The instruction set that is available to the user is determined when

the monitor is compiled. The choice of the values of DEVICE, EMULATOR

and COPROCESSOR determine which instruction set is available.

See the "Target System Configuration" section below.

CAUTION:  This assembler/disassembler does not insert code into the source

program; it merely overwrites memory at that location.  As a

result, a program patch that requires code insertion can be

accomplished by first Block Moving code to free up an insertion

area and then inserting into that area.

3.2     ENTERING AND MODIFYING SOURCE PROGRAM

In order to enter and modify source code, the as command should be executed as

detailed in section 2.2 (ie. as <ADDR>).  This places the user into the

Assembler/Disassembler routine.  Table 3.1 summarizes the commands that can

be executed within this routine:

TABLE 3.1  ASSEMBLER/DISASSEMBLER SUB COMMANDS

__________________________________

Command                 Designator

__________________________________

BACKUP <EXP>                    - <EXP>

ADVANCE <EXP>                   + <EXP>

FINISH                          .

HELP                            ?

STEP PAST               carriage return

DEFINE CONSTANT		    DC #<EXP>

Note: Executing a '?' while in the assembler/disassembler returns the DEVICE

that the assembler/disassembler is supporting.

3.3     ENTERING A SOURCE LINE

After executing an as <ADDR> command, the assembler/disassembler

returns with the disassembly of the code found at that location.  At this

time the user may execute an assembler command shown in section 3.2 or type in

the source line that is to replace the displayed source code.  While entering

source the standard MOTOROLA effective addressing modes are used.  These modes

are summarized below in Table 3.2:

TABLE 3.2  ASSEMBLER/DISASSEMBLER EFFECTIVE ADDRESSING MODES

Effective Addressing Mode                               Syntax

Register Direct                                 Dn

Address register direct                         An

Address register indirect                       (An)

Address register indirect

with Postincrement                    (An)+

Address register indirect

with Predecrement                     -(An)

Address register indirect

with Displacement                     (d16,An)

Address register indirect

with Index (d8)                       (d8,An,Xn.SIZE*SCALE)

Address register indirect

with Index (base displacement)        (bd,An,Xn.SIZE*SCALE)

Memory indirect Post-indexed                    ([bd,An],Xn.SIZE*SCALE,od)

Memory indirect Pre-indexed                     ([bd,An,Xn.SIZE*SCALE],od)

PC indirect with displacement                   (d16,PC)

PC indirect with index (d8)                     (d8,PC,Xn.SIZE*SCALE)

PC indirect with index (bd)                     (bd,PC,Xn.SIZE*SCALE)

PC memory indirect Post-indexed                 ([bd,PC],Xn.SIZE*SCALE,od)

PC memory indirect Pre-indexed                  ([bd,PC,Xn.SIZE*SCALE],od)

Absolute Short Address                          (xxx).W

Absolute Long Address                           (xxx).L

Absolute Address                                xxx optimizes (bwl)

Immediate Data                                  #xxx

While using the POST or PRE indexed modes, fields may be skipped by using a

comma.  An example is shown below:

Ex.     andi #12,([,],,)                ====>

andi.b #$12,([$0,ZA0],ZD0.W*1,$0)

Other examples of source lines are shown below:

Ex.     ori.l  #12,(a1)                 =====>  ori.l #$12,(a1)

Ex.     addq    #1,(a1)                 =====>  addq.b #$1,(a1)

There are only limited error screening abilities included within the monitor.

Examples of this are shown below:

Ex.     jmp (123).w                     ====>   jmp ($123).W

Note: 	When executed results in a bus error.

Ex. 	bsr (123) ====>

ERROR 10:illegal change of flow ===> bsr (123)

Note: 	The bsr instruction does check for illegal

changes of flow.

NOTE:	Flow may not be changed to an odd addr.

Upper digits of data are NOT truncated when a mismatch between size and

immediate data is found if the byte or word size option was specifically

entered. If the long size option is specified and data exceeds this range then

upper digits ARE truncated.

Ex.	addi.b #12345678,(a1) ====>

ERROR 11:immediate data/size option error ===> addi.b #12345678,(a1)

Ex.	addi.l #123456789,(a1) ====>  addi.l #$23456789,(a1)

Ex.	addi   #123456789,(a1) ====>  addi.l #$23456789,(a1)

(defaults to the long option)

truncated--------------^

Upper digits of data are truncated on commands that have a limited field

in the opword to store the immediate data. Examples of this are shown below.

Ex.     addq    #10,(d0)        =====>  addq.b #$0,(d0)

Ex.     trap    #10             =====>  trap #$0   input is hex default

Ex.     trap    #&10            =====>  trap #$A

3.4     ENTERING CHANGE OF FLOW INSTRUCTIONS

Since the assembler/disassembler does not use labels, all instructions that use

<label> as an effective addressing mode must have their displacement determined.

If initially unknown,  space for this displacement must be reserved and then the

user needs to come back and enter the displacement.  Once the displacement

has been determined it may be entered as shown in the following example:

Ex. In this example the location of the target instruction of a branch is known

to be $100000 and a BRA is needed at location 0.  After executing "AS 0" and

obtaining the disassembly found at 0 the user could type:

BRA     100000          or

BRA     (100000)        or

BRA.l   #ffffe

The absolute addressing mode can be used if the target address of a branch is

known (as in the first 2 examples) or the displacement (last example) can be

entered using the immediate data addressing mode.

CAUTION:  	In some instances unexpected results can occur while using

change of flow statements.  These instances are summarized below

with examples.

Ex. 1  If the degenerate case of a branch statement in used (ie. attempting to

use a short branch to branch to the following instruction) the assembler

mistakenly assembles this .b option.  However, since the displacement is zero

this is a .w opcode and the disassembler correctly displays this fact to the

user.

$00004000       nop  ? bra.b  ====> results in an INCORRECT assembly

Ex. 2  If the user attempts to force a particular size branch statement and the

actual branch requires a greater displacement than was reserved then the

assembler prints an error message: "ERR0R 16:  OUT OF DISPLACEMENT RANGE" .

$00004000       nop  ?bra.w (100000)

One way to assure this does not occur is to not enter a size option.  This

allows the assembler to choose the correct size for the displacement.

3.5 ENTERING REGISTERS and REGISTER LISTS

The move multiple register instruction (movem) uses a register list as an

effective address.  This list may be entered in the following method:

Ex.

a0                      ====>   single address register

d3                      ====>   single data register

a0-a3                   ====>   series of registers

a0-a3/a7                ====>   combination of previous examples

a0-a7/d0-d7             ====>   all of the registers

If coprocessor support is specified  then the floating point registers

can be entered as shown below:

Ex.

fp0                      ====>   single floating point register

fp0-fp2                  ====>   series of registers

fp0-fp3/fp7              ====>   combination of previous examples

Many of the commands require the entering of registers other than data or

address registers.  Tables 3.3-8 show listings of the registers that are used

and the abbreviations accepted by the assembler:

TABLE 3.3  68000/68HC000/68008 REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

User Stack Pointer                      USP

Program Counter                         PC

Condition Code Register                 CCR

Supervisor Stack Pointer                SSP

Status Register                         SR

TABLE 3.4  68010 REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

User Stack Pointer                      USP

Program Counter                         PC

Condition Code Register                 CCR

Supervisor Stack Pointer                SSP

Status Register                         SR

*Vector Break Register                  VBR

*Source Function Code Register          SFC

*Destination Function Code Register     DFC

Note: * Represents a change from the previous

model

TABLE 3.5  68020 REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

User Stack Pointer                      USP

Status Register                         SR

Condition Code Register                 CCR

Program Counter                         PC

*Master Stack Pointer                   MSP

*Interrupt Stack Pointer                ISP

Vector Break Register                   VBR

Source Function Code Register           SFC

Destination Function Code Register      DFC

*Cache Control Register                 CACR

*Cache Address Register                 CAAR

_____________________________________________

TABLE 3.6  68030 REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

User Stack Pointer                      USP

Status Register                         SR

Condition Code Register                 CCR

Program Counter                         PC

Master Stack Pointer                    MSP

Interrupt Stack Pointer                 ISP

Vector Break Register                   VBR

Source Function Code Register           SFC

Destination Function Code Register      DFC

Cache Control Register                  CACR

Cache Address Register                  CAAR

*CPU Root Pointer Register              CRP

*Supervisor Root Pointer Register       SRP

*Translation Control Register           TC

*Transparent Translation Register 0     TT0

*Transparent Translation Register 1     TT1

*MMU Status Register                    PSR

_____________________________________________

TABLE 3.7  68040 REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

User Stack Pointer                      USP

Status Register                         SR

Condition Code Register                 CCR

Program Counter                         PC

Master Stack Pointer                    MSP

Interrupt Stack Pointer                 ISP

Vector Break Register                   VBR

Source Function Code Register           SFC

Destination Function Code Register      DFC

Cache Control Register                  CACR

Cache Address Register                  CAAR

User Root Pointer Register              URP

Supervisor Root Pointer Register        SRP

Translation Control Register            TC

*Data Transparent Translation Register 0 DTT0

*Data Transparent Translation Register 1 DTT1

*Instruction Transparent Trans. Reg.   0 ITT0

*Instruction Transparent Trans. Reg.   1 ITT1

MMU Status Register                     PSR

Note: Also the floating point registers below

_____________________________________________

TABLE 3.8  FLOATING POINT REGISTERS

____________________________________________

Name                                    Syntax

____________________________________________

Floating Point Control Register         FPCR

Floating Point Status Register          FPSR

Floating Point Inst. Address Register   FPIAR

Floating Point Data Register            FP0-FP7

NOTE:	PMMU (68851) support is not available under release 1.0.

3.6 ENTERING/EVALUATING FLOATING POINT IMMEDIATE DATA

Floating point immediate data must be entered using a decimal point with at

least one (1) digit in front of the decimal place (even if it is a '0').

Ex. 0.0012.  Since the C compiler used was not based on the draft proposed

version of ANSI C the software is incapable of performing the 'assembling' of

extended immediate data to extended precision. The monitor makes the correct

conversion up to double precision and places this result in an extended

format.  If the compiler that is being used does conform to allowing a

'long double' type then changing the type of the variable 'weight' in the

EAallowed routine (in the asm68.c file) from double to long double should

provide the added precision.  Examples of floating point immediate data shown

below:

Ex.  fmove.s #5.0,fp0	====> fmove.s 1_400000_E_2,FP0

The format on the disassembly is integer part_fraction field_E_exponent field,

where the fraction bits represent weighting of 1/2 ,1/4,....etc. from the left

to the right.  The exponent bits represent the unbiased power that 2 should be

raised to. A conversion to decimal can be accomplished by evaluating:

integer + evaluated fraction  x 2^exponent field

In the above example this equates to:

(1 + .25) x 2^2 = 5.0

NOTE:  The monitor uses the round toward zero rounding mode in the assembler

when assembling floating point immediate data.

3.7 ENTERING MMU INSTRUCTIONS

MMU instruction should NOT be entered with a size descriptor.  The assembler

defaults to the correct size.

Ex. pmove (a0),tt1 	;asssembles

pmove.l (a0),tt1 	;does not assemble even though the operation

;is a long operation.

A summary of the mnemonic representation of the MMU registers can be found in

Table 3.5 and 3.6 for the 68030 and 68040 monitors respectively.

CHAPTER 4

SOFTWARE MODIFICATIONS

4.1     SOFTWARE OVERVIEW

The FBUG monitor uses 4 header files.  These header files are briefly described

below.

TARGETSYS.H

This file is the header file that is used to configure the monitor to the

target system. All of the predefined variables that are used by the monitor

are found here.  It is accessed when any call to #include "userdef.h" is

made.

USERDEF.H

This is the file that defines all of the structures that are used by

the monitor.

TEXTDEF.H

This file has all of the strings that are used by the monitor.  It is also

where most of the structures that are used by the monitor are implemented.

VARDEF.H

This file has all of the global variables that are used by the monitor.

The flow of the FBUG monitor is outlined below:

asmstartport.s ===> coldstartport.c ===> main.c ----|

^-------|

The first source file (asmstartport.s) initializes the microprocessor to a

known state and then branches to the main{} statement in the coldstartport.c

routine.  The coldstartport.c routine performs all of the initialization for the

monitor before branching to the mainloop{} routine in the main.c file.  The

mainloop{} routine is a continuous loop used by the monitor.  From this loop

the various commands of the monitor are called.

4.2     ADDING/DELETING COMMANDS

The process of adding or deleting commands from the monitor is a relatively

simple process.  Adding a command involves making 2 additions to the textdef.h

header file and then developing/including the command description file with the

monitor.

The two changes that need to be made to the textdef.h header file are:

1.) include the function name that implements the command as an externally

defined routine. For example the line in the textdef.h file that states

extern trcmd(),pfcmd(),ascmd(),sdcmd(),ctcmd();

accomplishes this for those five commands.

2.) include the command in the f[] structure with the appropriate information.

For an example of how this is done the "ct" command entry is discussed:

The line

{"ct Confidence Test \n",ctcmd,"ct \n"}

includes the ct command in the f[] structure.  The first textstring (ie. ct

Confidence Test \n) is the string that is used for the command listing print

out performed when the help or ? commands are executed.  Also, the first word

of this string is the command name that is searched for when attempting to

execute this command.  The function name "ctcmd" is the name of the function

that is executed when this command is used by the monitor.  The last text

string (ie. ct \n) is the string that describes the type of syntax used with

this command.

Next, the command description file has to be written and included in the

makefile of the monitor (see CHP6 LINKING INFORMATION for more information

concerning the linking process). The calling convention for the functions which

perform the command is to pass the function two parameters. These two parameters

are argc and argv.  They are the number of arguments on the command line and the

actual command line respectively.

Deleting a command is accomplished by following an analogous path. Removing

the 2 lines that incorporate a command into the monitor and removing the command

from the linking process effectively remove the command from the monitor.

4.3     SWITCHING DRIVERS

The I/O driver used by the monitor in version 1.0 is for a MC68681 DUART. If the

target system uses some other driver, then the following changes needs to

be made:

1.)     The DUART initialization performed in coldstartport.c has to be

modified to accomplish the same results with the new driver.

2.)     The user definitions associated with the new driver has to be

updated in the targetsys.h file.  (ie. the offsets to the various

buffers of the new driver)

3.)     It may be necessary to modify the getch and putch routines found in

getline.c and print.c respectively.

CHAPTER 5

TARGET SYSTEM CONFIGURATION

5.1 INTRODUCTION

The FBUG monitor can be configured to support any of the 68xxx microprocessors.

This is accomplished by a series of define statements found within the

"targetsys.h" header file.  By defining which configuration is supported

additional software can be excluded from the code at compilation time to

result in a smaller monitor. These space savings are generally accomplished

based on the size of the assembler/disassembler for the associated

configuration.  The monitor can also be configured to a specific memory

configuration.

5.2   ASSEMBLER/DISASSEMBLER

The assembler/disassembler supported in the 'AS' command is determined by the

definiton of the DEVICE, EMULATOR and COPROCESSOR variables found in the

targetsys.h header file. The choices for DEVICE are 68000,68008,68010,68020,

68030 or 68040. This also determines the programmer's model that is supported in

the 'RD' and 'RM' commands. If the DEVICE selected is '68040' then the EMULATOR

define is used to determine whether 68881/68882 commands not implemented in

hardware should be supported. Placing a 'TRUE' here stipulates that all

68881/882 instructions should be supported. If there is no software emulation of

these commands then a 'FALSE' should be placed here. If the DEVICE selected is

either 68020 or 68030 then the COPROCESSOR (TRUE or FALSE) define statement

determines whether the 68881/882 instruction set should be supported.

Note: As of release 1.0 the 68040 register model is visible but supervisor

registers other than those implemented on the 68020 are not supported.

As a result of this the monitor displays the entire 68040 register set

and APPEARS to allow the user to modify these registers with a RM command.

However, when executing target code (GO or TR command) the monitor does

NOT use these values to intialize the state of the microprocessor

before executing the target code.  In addition, these registers are NOT

captured by the monitor if they have been modified by target code.

5.3	MEMORY REQUIREMENTS

The "targetsys.h" file contains all of the variables that determine the

target system configuration.  These variables are listed below and

needs to be changed to reflect the configuration of the target

system.

/* ******************************************************************** */

/* ********************** MEMORY MAP FOR DEBUGGER ********************* */

#define DUARTALOC 0xffe02000	/* MC68681 duart channel a */

#define DUARTBLOC 0xffe02008	/* MC68681 duart channel b */

#define ROMLOC 0x100000		/* system ROM space			*/

/* .text segment should be loaded here	*/

/* .data segment should be loaded here	*/

#define SYSRAMLOC  0x000fe000

/* system RAM space 			*/

/* NOTE: space must be reserved for the

location of the .bss used by

the monitor.			*/

/* Provided the .bss is not larger than

1000 (ie. VBRLOC resides at SYSRAMLOC

+ 1000) placing the .bss from the

monitor at this location is allowed 	*/

#define ISPLOC SYSRAMLOC+0x2000 /* defines for monitor 	*/

#define MSPLOC SYSRAMLOC+0x1c00

#define USPLOC SYSRAMLOC+0x1b00

#define VBRLOC SYSRAMLOC+0x1000

#define USERPCLOC 0x4000 	/* defines for usercode	*/

#define USERVBRLOC 0x0

/* *********************************************************************** */

#if(DEVICE<68020)

#define USERSSPLOC USERVBRLOC+0x2000	/* Note: Only one of these is

used based on the device

being implemented */

#else

#define USERISPLOC USERVBRLOC+0x2000

#endif

/* *********************************************************************** */

#define USERMSPLOC USERVBRLOC+0x1c00

#define USERUSPLOC USERVBRLOC+0x1b00

#define SRSTART 0x2700		/* status register	*/

******************************************************************************

The DUART assumed is a MC68681 and its respective channels are found

at the stated locations.  If this is not the case, then the initialization

routine in coldstartport.c has to be changed.

The next 5 locations describe the memory map of the system used for the port.

The next location describes the value that is placed in the VBR register.

NOTE: 	This is the location of the vector offset table. This space is used

by the monitor to set up the offset table.

The next 5 locations describe the initial values that the monitor

uses for "user code".

The next location describes the value that is placed in the user's VBR

register.

NOTE: This is the location of the user's vector offset table and is

also initialized by the monitor.

SRSTART is the initialized value of the  MC68xxx Status Register

CHAPTER 6

LINKING INFORMATION

6.1 INTRODUCTION

The FBUG monitor is a stand alone software package written in C

and developed under the UNIX environment.  The combined .text and .data

section of the object file varies from approx. 54K to 87  bytes of memory.

Different configurations are summarized below in Table 6.1.  The purpose of this

chapter is to describe the method to compile and link the various files that

make up the FBUG monitor.  There is also a brief description of the necessary

changes that need to be made in order to port the FBUG monitor to a MC68xxx

based board.

TABLE 3.3  68000/68HC000/68008/68010 REGISTERS

____________________________________________

DEVICE	COPROCESSOR	EMULATOR	SIZE (KB)

____________________________________________

68000/008/010	----NA-----	---NA---	54K

68020	      FALSE	---NA---	61K

68020	       TRUE	---NA---	81K

68030	      FALSE	---NA---	65K

68030	       TRUE	---NA---	87K

68040	----NA-----	   FALSE	82K

68040	----NA-----	    TRUE	84K

NOTE: 	----NA---- represents a don't care

for that particular DEVICE.

____________________________________________

6.2	DESCRIPTION OF FBUG(68xxx) MAKEFILE

The makefile used to create the FBUG monitor under the UNIX

operating system is shown below and can be found in the file named

"makefile" in this directory.

MAKEFILE

portbug : coldport.o main.o printport.o getlineport.o general.o \

asmprocport.o \

bf.o bm.o br.o bs.o dc.o he.o lo.o rd.o rm.o tm.o \

mm.o md.o go.o tr.o sd.o as.o asm68.o disasm68.o asmcode68.o \

asmhandler.o handler.o  asmstartport.o conf.o fproutines.o 68def.o

ld ./ifilemb  asmstartport.o asmhandler.o coldport.o \

printport.o getlineport.o general.o asmprocport.o main.o bf.o bm.o \

bs.o dc.o he.o lo.o rd.o rm.o tm.o mm.o md.o go.o tr.o br.o \

sd.o as.o disasm68.o asm68.o asmcode68.o handler.o conf.o \

fproutines.o  68def.o \

-m -o fbugxxxd >memmap

ubuilds fbugxxxd

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

NOTE: In fbugxxxd     xxx ===> 000,008,010,020 etc...

d ===> T or F for whether Coprocessor supported

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

main.o :	main.c userdef.h textdef.h vardef.h

:

:

:

:

:

asmproc.o : asmproc.c userdef.h textdef.h vardef.h

asmstartport.o : textdef.h vardef.h userdef.h targetsys.h

/lib/cpp asmstartport.s  > temp.s

as  -o asmstartport.o temp.s

rm temp.s

asmcode68.o : textdef.h vardef.h userdef.h targetsys.h

/lib/cpp asmcode68.s  > temp.s

as  -o asmcode68.o temp.s

rm temp.s

asmhandler.o : textdef.h vardef.h userdef.h targetsys.h

/lib/cpp asmhandler.s  > temp.s

as  -o asmhandler.o temp.s

rm temp.s

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

NOTE: 	In order to ease the porting of the monitor to a

specific system, a C preprocessor is executed on the

three (3) .s files above.  This performs a text

substitution into these files from the targetsys.h

header file.  If the development system being used does

not support this capability then these text substitutions

must be made manually.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Executing the "make" command under UNIX with this makefile

produces the file "fbug.mx" which is the stand alone monitor.

The make command performs the following actions:

1.	Checks to see if all of the current .o files on which

fbugxxxd depends are current.  If not then compile the

source files to create those object files.  It also checks

to verify that the files on which these files depend on

are up to date.  If they are not then they are

compiled first.

/* ******************************!!!!!!!!!********************************* */

NOTE: These dependencies are not conclusive and if a change is made to any of

the header files then all of the files should be recompiled until

a thorough understanding of the interdependencies between the modules

is achieved.

/* ******************************!!!!!!!!!********************************* */

2.	After all the required .o files have been brought up to

date they are linked together in the order described

after the "ld" command.  The "ifilemb" file is used as a ld

directive to describe the memory configuration of the

target system.  In this instance, the target system has

the following memory configuration:

00000000-00000255	RAM

000FE000-00100000	RAM

00100000-00110000	ROM

The ifilemb informs the UNIX linker of this target

configuration and also where to place each section.  In

this instance the .text section is being placed into ROM

and the .data and .bss section are being placed in RAM.

Refer to the "ifilemb" file in this directory.

/* ******************************!!!!!!!!!********************************* */

NOTE: Space has to be made available for both the monitor's vector offset

table and the user's vector offset table. (ie. 256 32-bit words has

to be available at locations VBRLOC and USERVBRLOC.

(ie.  00000000-00000255	RAM)

NOTE: The monitor uses approx. $1000Kb ram for the .bss variables (placed

at SYSRAMLOC). Next the vector offset table is placed at SYSRAMLOC +

$1000. Then the stack space used by the monitor is placed above this.

Using the default configuration provided requires that approx. $2500

bytes of RAM be reserved for the monitor begining at location SYSRAMLOC

and that 256 32-Bit words be reserved at the USERVBRLOC (which

is at location 0 in the released version).  If user code is run then

the user is responsible for making sure that the stack space used by

the user does not interfer with the monitors space.

/* ******************************!!!!!!!!!********************************* */

3.	These files are now linked and this file is labeled "fbugxxxd"

4.	Next, the "ubuilds fbugxxxd" command creates an S-Record

from the fbug file that was previously created.  This S-

Record can be used to download to systems that have the

capability to load S-Records.

The environment that is being worked under has to be able to perform the

functions stated above in order for the user to create their FBUG object file.

6.3	MONITOR USED TO BOOT THE SYSTEM

If it is desired for the monitor to be at the reset location (ie. logical

0 in memory)  then the following two lines should be added above

the start: routine in the asmstartport.s file:

INITSP:	long 0x100000

INITPC	long start

NOTE:  The ifilemb directive must still accurately reflect the

location of ROM on the system.